Elements Of Computing System - 1

End Semester Project Report

Submitted By:

Mettu Siddhartha- BL.EN.U4AIE21080                    Group: E6

In partial fulfillment for the award of the degree

Of

BACHELOR OF TECHNOLOGY

IN

ARTIFICIAL INTELLIGENCE

AMRITA SCHOOL OF ENGINEERING, BANGALORE

AMRITA VISHWA VIDYAPEETHAM

BANGALORE –560035

FEB 2022

Faculty: Ms. Meena Belwal                                  Date: February 16,2022

Question-1

Implement the four Boolean functions listed using three half-adder circuits.

D = A xor B xor C

E = A'BC+ AB'C

F = ABC' + (A'+B') C

G = ABC

Truth Table

| a | b | c | d | e | f | g |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 |

CODE:

CHIP ThreeHalfAdders{

IN  a, b, c;

OUT sum,

carry;

PARTS:

HalfAdder(a=a,        b=b,    sum=axorb,          carry=ab);

HalfAdder(a=axorb, b=c,    sum=d,   carry=e);

HalfAdder(a=c,        b=ab,  sum=f,        carry=g);

}

Question-2

Explain the 4-bit binary up counter. Implement it using T Flip Flop.

Truth Table

| T | Qn | Qn+1 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |